BC Softsyn Synthesizer

Comprehensive Reference Guide

Software & Hardware Synthesizer Architecture, Design, and DSP

# 1. Types of Software and Hardware Synthesizers

Synthesizers can be categorized by their sound generation method, signal flow, and form factor. The following covers both historical and contemporary types found in hardware instruments and software plugins.

## 1.1 Subtractive Synthesizers

The most common synth architecture. Starts with a harmonically rich waveform from an oscillator, then uses filters to subtract (remove) partials, shaping the timbre.

- Hardware examples: Minimoog Model D, Roland Juno-106, Korg MS-20, ARP Odyssey, Sequential Prophet-5

- Software examples: Arturia Mini V, u-he Diva, Korg Legacy MS-20, Native Instruments Massive

- Best for: bass lines, leads, pads, classic analog sounds

## 1.2 Additive Synthesizers

Builds sound by adding together sine waves (partials) at different frequencies and amplitudes, inspired by Fourier analysis. Gives precise spectral control but requires many oscillators.

- Hardware examples: Hammond organ (tonewheel additive), Kawai K5000, Synclavier

- Software examples: Native Instruments Razor, Camel Audio Alchemy (additive mode), Image-Line Harmor

- Best for: bell tones, organ-like timbres, evolving spectral pads, formant sounds

## 1.3 FM (Frequency Modulation) Synthesizers

Uses one oscillator (modulator) to modulate the frequency of another (carrier), creating complex sidebands and harmonic content from simple waveforms. Pioneered by John Chowning and commercialized by Yamaha.

- Hardware examples: Yamaha DX7, DX21, TX81Z, Korg Volca FM, Elektron Digitone

- Software examples: Native Instruments FM8, Plogue Chipsynth, Dexed (free DX7 emulation)

- Best for: electric pianos, bells, metallic sounds, bass, percussive tones

## 1.4 Wavetable Synthesizers

Stores short single-cycle waveforms (wavetables) in memory and scans through them for evolving timbres. PPG Wave pioneered this concept.

- Hardware examples: PPG Wave 2.3, Waldorf Blofeld, Ensoniq VFX, Korg Wavestate

- Software examples: Xfer Serum, Native Instruments Massive X, Waldorf Nave, Vital

- Best for: evolving pads, digital leads, morphing textures, modern EDM sounds

## 1.5 Sample-Based / ROMpler Synthesizers

Uses digitally recorded audio samples as sound sources. PCM (Pulse-Code Modulation) samplers store real instrument recordings and play them back pitched across a keyboard.

- Hardware examples: Roland D-50, Korg M1, Yamaha Motif, Kurzweil K2600, Roland Fantom

- Software examples: Native Instruments Kontakt, Roland Cloud, Spectrasonics Omnisphere (sample layer)

- Best for: realistic instrument emulation, cinematic orchestral sounds, hybrid textures

## 1.6 Physical Modeling Synthesizers

Simulates the acoustic physics of real instruments using mathematical models (waveguide synthesis, finite-difference methods). Produces highly realistic and playable results.

- Hardware examples: Korg Prophecy, Yamaha VL1, Roland V-Piano

- Software examples: Applied Acoustics Systems Chromaphone, Modartt Pianoteq, Korg Wavestation (DSP)

- Best for: acoustic instruments, plucked strings, wind instruments, experimental timbres

## 1.7 Granular Synthesizers

Breaks audio into tiny grains (typically 1–100 ms) and manipulates their playback speed, pitch, position, and density. Produces lush, cloud-like textures.

- Hardware examples: Critter & Guitari Organelle (with patches), Tasty Chips GR-1, Polyend Medusa

- Software examples: Native Instruments Stutter Edit 2, Arturia Pigments (granular mode), u-he Uhbik-G

- Best for: ambient textures, time-stretching, glitchy soundscapes, drone pads

## 1.8 Vector Synthesizers

Blends between multiple sound sources using a joystick or XY pad. Popularized by Sequential Prophet VS and Korg Wavestation.

- Hardware examples: Sequential Prophet VS, Korg Wavestation, Yamaha SY22/TG33

- Software examples: Korg Legacy Wavestation, TAL-U-No-62

- Best for: evolving pads, cinematic transitions, animated timbres

## 1.9 Semi-Modular Synthesizers

Pre-wired signal paths with patch points that allow reconfiguration. Bridges fully modular and fixed-architecture designs.

- Hardware examples: Moog Grandmother, Moog Mother-32, Make Noise 0-Coast, Arturia MiniBrute 2

- Software examples: Softube Modular (within DAW), VCV Rack

## 1.10 Modular Synthesizers (Eurorack/Buchla/5U)

Fully patch-programmable systems with no fixed signal path. Modules connect via patch cables. Three primary formats: Eurorack (3.5mm jacks, ±12V), Buchla (banana jacks, idiosyncratic), 5U/MU (quarter-inch jacks).

- Hardware examples: Any Eurorack system (Make Noise, Mutable Instruments, Intellijel), Buchla 200e, Serge

- Software examples: VCV Rack (free), Softube Modular, Reaktor Blocks

- Best for: experimental sound design, generative music, academic/research synthesis

## 1.11 Virtual Analog (VA) Synthesizers

Uses DSP to emulate analog circuitry, including oscillator drift, filter saturation, and component-level behavior. See Section 4 for detailed VA discussion.

- Hardware examples: Clavia Nord Lead, Access Virus, Roland JP-8000, Waldorf Q

- Software examples: Arturia V Collection, u-he Diva, TAL-U-No-LX

## 1.12 Hybrid Synthesizers

Combines multiple synthesis methods in one instrument, e.g., samples with analog filters, or FM with subtractive.

- Hardware examples: Roland JD-800, Korg Kronos, Spectrasonics Omnisphere 2 (hardware controller)

- Software examples: Spectrasonics Omnisphere 2, Arturia Pigments

## 1.13 Spectral / Resynthesis Synthesizers

Analyzes the spectral content of audio using FFT, then resynthesizes it. Allows radical manipulation of real sounds.

- Software examples: Image-Line Harmor, camel Audio Alchemy, Bitwig Spectral Suite

- Best for: morphing real sounds, pitch-shifting without artifacts, spectral filtering

## 1.14 String Machines & Ensemble Synthesizers

Dedicated to emulating string orchestra sounds using divide-down oscillators and ensemble chorus effects.

- Hardware examples: ARP String Ensemble, Roland RS-09, Solina String Ensemble

- Software examples: Arturia Solina V, GForce OB-E

# 2. Building Blocks of Synthesizers

Every synthesizer—analog or digital—is assembled from a set of fundamental signal-processing building blocks. Understanding these components is essential to designing and programming any synthesis system.

## 2.1 VCO – Voltage Controlled Oscillator

The primary sound source. Generates a continuous waveform whose pitch is controlled by voltage (analog) or by MIDI/digital data (DCO/NCO in digital synths). See Section 3 for oscillator types.

## 2.2 DCO – Digitally Controlled Oscillator

An oscillator whose pitch is controlled by a digital counter, providing more tuning stability than pure VCOs. Used in Roland Juno series. Still produces analog waveforms.

## 2.3 VCF – Voltage Controlled Filter

Shapes the harmonic content of the oscillator output by attenuating frequencies above, below, or around a cutoff frequency. See Section 4 for filter types.

## 2.4 VCA – Voltage Controlled Amplifier

Controls the output volume of a signal path using a control voltage, typically driven by an envelope. Sets the amplitude over time (attack, decay, sustain, release).

## 2.5 EG / ADSR – Envelope Generator

Generates a time-varying control signal with up to four stages: Attack, Decay, Sustain, Release. Applied to VCF cutoff, VCA level, pitch, and other destinations.

- ADSR (standard): Attack, Decay, Sustain, Release

- ADBDSR: Attack, Decay 1, Break, Decay 2, Sustain, Release — used in Yamaha DX7

- AD: Simple two-stage (percussion)

- DAHDSR: Delay, Attack, Hold, Decay, Sustain, Release — extended version

- Multi-stage envelopes: any number of segments with custom slopes

## 2.6 LFO – Low Frequency Oscillator

Oscillates below audible range (typically 0.1–30 Hz) to modulate other parameters cyclically. See Section 5 for LFO types.

## 2.7 Mixer / Signal Combiner

Sums or blends multiple signals (oscillators, audio sources, modulation). Can apply level and panning to each input.

## 2.8 Noise Generator

Produces spectrally dense signals useful for percussive attacks, wind sounds, and as modulation sources.

- White noise: equal energy per frequency

- Pink noise: -3 dB/octave roll-off

- Red/Brown noise: -6 dB/octave roll-off

- Blue/Violet noise: rising high-frequency content

## 2.9 Ring Modulator

Multiplies two audio signals together, producing sum and difference sidebands. Creates metallic, bell-like tones. Used in Moog and EMS synthesizers.

## 2.10 Sample & Hold (S&H)

Samples an input signal (often noise) at regular or triggered intervals and holds that value until the next sample. Creates random stepped modulation.

## 2.11 Slew Limiter / Portamento

Limits the rate of change of a signal, creating smooth glides between pitches (portamento/glissando) or smoothing stepped modulation.

## 2.12 Waveshaper / Wavefolder

Distorts the shape of a waveform, adding new harmonics. Wavefolding reflects portions of the waveform back when they exceed a threshold, creating complex harmonic content.

## 2.13 Clock / Timing Source

Provides a rhythmic pulse to drive sequencers, LFOs, S&H, and other time-based components. Can be internal or external (MIDI clock, analog clock).

## 2.14 MIDI Interface / CV/Gate

Translates performance data (keyboard, controllers) into control signals for the synthesis engine. MIDI is digital; CV/Gate is analog (CV = pitch in volts/octave, Gate = on/off trigger).

## 2.15 Effects Processor

Post-processing modules that add spatial, temporal, or spectral treatments. See Section 6 for types.

## 2.16 Modulation Matrix

A routing system that connects any modulation source to any destination with variable depth. Common in software synths like Serum, Massive, and Omnisphere.

## 2.17 Arpeggiator

Plays notes of a held chord in sequence automatically. See Section 9 for types.

## 2.18 Sequencer

Records and plays back a series of notes or parameter values in order. See Section 8 for types.

# 3. Types of Oscillators

Oscillators are the primary sound sources in a synthesizer. They generate periodic waveforms or complex signals that are then shaped by filters and amplifiers.

## 3.1 Waveform Types

- Sine wave: Fundamental frequency only, no harmonics. Pure, smooth tone. Used in FM carriers and bass fundamentals.

- Sawtooth (saw) wave: All harmonics, falling amplitude (1/n). Bright, buzzy sound. Ideal for strings, leads, bass.

- Reverse sawtooth (ramp): Opposite phase to saw. Slightly different tonal character.

- Square wave: Odd harmonics only. Hollow, clarinet-like. Width variable (see pulse wave).

- Pulse wave: Square wave with variable duty cycle (pulse width). Wider = thinner, narrower = buzzier.

- Triangle wave: Odd harmonics with faster rolloff (1/n²). Soft, flute-like. Halfway between sine and square.

- Super saw: Multiple detuned saw waves summed. Used in trance, EDM supersaws (Roland JP-8000 popularized this).

- White / pink noise: Used as oscillator source in some synths for atonal textures.

## 3.2 Oscillator Architectures

- VCO (Voltage Controlled Oscillator): True analog, subject to thermal drift. Warm, organic. Minimoog, ARP 2600.

- DCO (Digitally Controlled Oscillator): Digitally tuned but analog waveform. Stable, less drift. Roland Juno-106.

- NCO (Numerically Controlled Oscillator): Pure digital, stable, arbitrary waveforms. Used in wavetable and VA synths.

- Phase accumulator oscillator: Accumulates phase digitally at a rate set by a frequency word. Standard in digital synthesis.

- Bandlimited oscillator (BLIT/BLEP/polyBLEP): Avoids aliasing by generating bandwidth-limited waveforms. Essential in VA synthesis.

- Wavetable oscillator: Reads through a table of single-cycle waveforms at a rate proportional to pitch.

- Additive oscillator bank: Multiple sine oscillators summed with independent amplitude/frequency control.

- FM operator: Sine wave oscillator that can modulate or be modulated by another operator.

## 3.3 Oscillator Modulation Techniques

- Pulse Width Modulation (PWM): LFO or envelope modulates duty cycle of a pulse wave. Creates chorusing/animation.

- Hard sync: Forces slave oscillator to reset its phase when master completes a cycle. Creates rich, bright harmonic content.

- Soft sync: Influences slave phase without full reset. Subtler tonal effect.

- Frequency Modulation (FM): One oscillator modulates the frequency of another. See FM synthesis.

- Phase Modulation (PM): Modulates phase directly; mathematically similar to FM but different tonal result.

- Ring modulation: Multiplies two oscillator outputs. Sidebands and sum/difference frequencies.

- Cross modulation (XMod): Bidirectional FM or AM between two oscillators.

- Wavefold: Folds the waveform back on itself for extreme harmonic generation.

- Detune / Spread: Multiple oscillators slightly detuned for chorus/thickening effect.

- Octave/interval stacking: Oscillators tuned at specific musical intervals (octave, fifth, etc.).

# 4. Types of Filters

Filters are the primary timbre-shaping element in subtractive synthesis. They attenuate specific frequency ranges defined by a cutoff frequency and slope (roll-off rate).

## 4.1 By Filter Response

- Low-pass filter (LPF): Passes frequencies below the cutoff; attenuates above. Most common in synthesis. Characteristic "wah" sweep.

- High-pass filter (HPF): Passes frequencies above the cutoff; attenuates below. Removes bass, thins the sound.

- Band-pass filter (BPF): Passes a band of frequencies around a center frequency; attenuates above and below.

- Band-reject / Notch filter: Attenuates a narrow band, passes everything else. Creates "hollow" or phased sounds.

- All-pass filter: Passes all frequencies but shifts their phase. Used in phasers.

- Peak / Bell EQ filter: Boosts or cuts around a center frequency.

- High-shelf / Low-shelf filter: Boosts or cuts all frequencies above or below a cutoff.

- Comb filter: Series of evenly spaced notches producing a comb-like frequency response. Creates flanging, resonant textures.

- Formant filter: Multiple band-pass filters tuned to mimic vowel formants of the human voice.

## 4.2 By Slope / Order

- 6 dB/octave (1-pole): Gentle rolloff. Rare in synthesis; used in simple tone controls.

- 12 dB/octave (2-pole): Moderate rolloff. Oberheim, Juno filter topology.

- 18 dB/octave (3-pole): Uncommon; found in some Korg designs.

- 24 dB/octave (4-pole): Steep rolloff. Classic Moog ladder filter topology. Dominant in subtractive synthesis.

- Higher order (48 dB+): Multiple cascaded filters; very steep. Found in digital/software synths.

## 4.3 By Analog Circuit Topology

- Moog ladder filter: Bob Moog's transistor ladder design. 4-pole LPF. Self-oscillation at maximum resonance. Warm, musical saturation.

- Sallen-Key filter: Op-amp-based topology. 2-pole. Used in Roland Juno, Oberheim SEM.

- State-variable filter (SVF): Simultaneous LP, HP, BP, Notch outputs from one circuit. Oberheim SEM, Korg MS-20 (stage 2).

- Steiner-Parker filter: HP, LP, BP available by routing. Aggressive character.

- Diode ladder filter: Uses diodes instead of transistors. Found in Roland TB-303, EMS VCS3. Gnarly, distinctive sound.

- OTA (Operational Transconductance Amplifier): CEM/SSM chips. Found in Prophet-5, Jupiter-8. Smooth resonance.

- Buchla LPG (Low Pass Gate): Combines LPF and VCA in a single circuit using vactrols. Natural, plucked character.

## 4.4 By Digital/Software Implementation

- Biquad IIR filter: Most common digital filter structure. Used in EQs and synthesis. Computationally efficient.

- Chamberlin state-variable filter: Efficient SVF implementation for digital audio.

- Zero Delay Feedback (ZDF) filter: Models analog filter feedback loop with zero samples of delay. More accurate than naive digital implementations. Used in u-he Diva.

- VA (Virtual Analog) filter: Models analog circuitry including nonlinearities and saturation. Trapezoidal integration methods.

- FIR (Finite Impulse Response) filter: Convolution-based, always stable, linear phase. Used for precision equalization.

- Convolution filter (impulse response): Applies a real room or hardware response via convolution.

## 4.5 Special Filter Types

- Vowel / vocal filter: Animates formant positions to create vowel-like sweeps ("aaah" to "eeeh").

- Resonator bank: Multiple narrow band-pass filters tuned to partials of a physical instrument.

- Morphing filter: Smoothly interpolates between different filter responses or topologies.

# 5. Types of LFOs (Low Frequency Oscillators)

LFOs operate below the audible range (typically 0.01–30 Hz) and are used as modulation sources. Their output cyclically varies a destination parameter over time.

## 5.1 LFO Waveform Shapes

- Sine: Smooth, cyclic modulation. Natural vibrato, tremolo, filter sweeps.

- Triangle: Linear ramping up and down. Similar to sine but with sharper peaks.

- Sawtooth (up): Linear rise, instant drop. Creates rhythmic ramp effects.

- Reverse sawtooth (down): Instant rise, linear fall. Opposite ramp effect.

- Square: Abrupt alternation between two levels. Trills, on/off pulsing.

- Pulse (variable width): Square with adjustable duty cycle. Rhythmic accents.

- S&H (sample and hold): Random stepped values. Creates random modulation, robotic effects.

- Random / smoothed random: Randomly walks between values with interpolation. Organic, chaotic movement.

- Exponential curves: Non-linear versions of standard shapes. More musical feeling.

- User-defined / custom: Draw or import arbitrary LFO shapes. Prevalent in modern soft synths like Serum, Vital.

## 5.2 LFO Operating Modes

- Free-running: LFO runs continuously, independent of note triggers. Phase varies per note.

- Synced to BPM: Rate quantized to note divisions (1/4, 1/8, 1/16, etc.). Rhythmically locked.

- Note-triggered (retrigger): LFO resets phase at each new note. Consistent per-note modulation.

- One-shot (single cycle): LFO runs through one cycle and stops. Functions like an envelope.

- Envelope follower mode: LFO tracks amplitude of an input signal.

- MIDI-synced: Phase locked to incoming MIDI clock.

## 5.3 LFO Destinations (Common)

- Pitch (vibrato): Cyclic pitch variation

- Filter cutoff (wah / auto-wah)

- Amplitude (tremolo)

- Pulse width (PWM)

- Pan position (autopan)

- Reverb send / effect parameters

- Wavetable position

- FM depth / operator level

## 5.4 Advanced LFO Features

- Phase offset: Multiple LFOs at offset phases for complex polyrhythmic modulation.

- Rate modulation: One LFO modulates another's rate (LFO-to-LFO).

- Depth envelope: LFO depth changes over time (e.g., vibrato that fades in after note attack).

- Tempo-divided LFOs: Different subdivisions allow polyrhythmic textures.

- Unipolar vs bipolar: Unipolar LFO swings 0 to +1; bipolar swings -1 to +1.

# 6. Types of Effect Processors

Effect processors modify the synthesized audio after it leaves the core synthesis engine. They add spatial, temporal, dynamic, and spectral treatments.

## 6.1 Time-Based / Spatial Effects

- Reverb: Simulates acoustic spaces by adding reflections and reverberation. Types: spring, plate, hall, room, convolution IR.

- Delay / Echo: Repeats the signal after a set time. Tap tempo, multi-tap, ping-pong, analog echo emulation.

- Chorus: Adds slightly detuned and delayed copies of the signal for a lush, widened sound.

- Flanger: Short delay modulated by an LFO, creating comb filtering and a jet-plane sweep.

- Phaser: All-pass filters create phase-shifted copy; summed with dry signal for notch sweeps.

- Stereo widener: Expands the stereo field using mid/side processing or Haas delay.

## 6.2 Dynamics Processors

- Compressor: Reduces dynamic range by attenuating signals above a threshold. Adds punch and sustain.

- Limiter: Hard ceiling on signal level. Prevents clipping.

- Gate / Expander: Attenuates signal below a threshold, reducing noise or creating choppy effects.

- Transient shaper: Independently controls attack and sustain envelope of a sound.

- Sidechain compression: Compression triggered by an external signal (e.g., kick drum ducking bass).

## 6.3 Distortion / Saturation Effects

- Overdrive: Soft clipping. Warm, tubey saturation.

- Distortion: Hard clipping. Aggressive, compressed harmonic content.

- Fuzz: Extreme clipping, often with asymmetric waveform shaping.

- Bitcrusher: Reduces bit depth and/or sample rate. Adds digital noise and aliasing artifacts.

- Waveshaper distortion: Applies a custom transfer function to the waveform.

- Tube/tape saturation: Emulates the nonlinear response of vacuum tubes or tape oxide.

- Amp simulation: Models guitar amplifier cabinet and speaker response (convolution or DSP).

## 6.4 Spectral / Frequency Effects

- Equalizer (EQ): Boosts or cuts specific frequency bands.

- Vocoder: Applies the spectral envelope of one signal (modulator) to the carrier (synth).

- Pitch shifter: Transposes audio by semitones or cents without changing tempo.

- Harmonizer: Adds pitched copies to the signal (e.g., +3rd, +5th harmony).

- Formant shifter: Shifts formant frequencies independently of pitch.

- Spectral processor (FFT-based): Freeze, smear, convolve, or filter in the frequency domain.

- Comb filter / resonator: Harmonic resonances at multiples of a base frequency.

## 6.5 Modulation Effects

- Tremolo: Amplitude modulated at LFO rate.

- Vibrato: Pitch modulated at LFO rate (as an effect, post-synthesis).

- Auto-pan: Amplitude-based stereo panning with LFO.

- Ring modulator (effect): Multiplies audio signal with a carrier oscillator.

- Frequency shifter: Shifts all frequencies by a fixed Hz amount (non-musical, unlike pitch shifting).

## 6.6 Utility / Special Effects

- Stereo imager: Mid/side manipulation to widen or narrow stereo image.

- Exciter / Aural enhancer: Adds high-frequency harmonics for air and presence.

- Noise gate: Silences signal below a threshold to reduce background noise.

- Lo-fi / Vinyl simulation: Adds noise, crackle, wow/flutter artifacts.

- Convolution processor: Applies any impulse response — reverbs, cabinet sims, effect models.

# 7. Types of Modulators

Modulators are signal sources that dynamically vary other parameters over time, creating movement, expression, and animation in the sound.

## 7.1 Envelope-Based Modulators

- ADSR envelope: Standard four-stage time-varying amplitude. Drives VCA, VCF, pitch.

- Multi-stage envelope: Arbitrary number of segments with variable slopes (attack, hold, decay 1, decay 2, sustain, release, etc.).

- Looping envelope: Cycles through stages repeatedly, acting like an LFO with custom shape.

- Velocity-sensitive envelope: Envelope depth scales with MIDI velocity.

- Pressure (aftertouch) envelope: Envelope triggered or modulated by aftertouch.

## 7.2 Performance Modulators

- MIDI velocity: Note-on velocity controls level, filter, envelope depth, etc.

- Aftertouch (channel/polyphonic): Pressure applied after key press; controls vibrato, filter, expression.

- Pitch bend wheel: Pitch shift up or down by a set semitone range.

- Modulation wheel (Mod Wheel / CC1): General-purpose modulation controller.

- Expression pedal (CC11): Continuous foot controller for volume or other parameters.

- Breath controller: Pressure sensor maps breath intensity to modulation.

- MPE (MIDI Polyphonic Expression): Per-note pitch bend, pressure, and slide. Used in Roli Seaboard, Linnstrument.

- CV (Control Voltage): Analog modulation signal in modular systems.

- Gate: On/off trigger signal; opens/closes envelopes.

## 7.3 Signal-Derived Modulators

- Envelope follower: Tracks the amplitude envelope of an audio input; outputs control signal.

- Pitch tracker / pitch-to-CV: Extracts pitch from audio and outputs corresponding control voltage.

- Frequency analyzer: Spectral analysis drives modulation (e.g., sidechain ducking by frequency band).

- Audio-rate modulation: Oscillator at audio frequency used as modulator (FM synthesis).

## 7.4 Algorithmic Modulators

- LFO (low frequency oscillator): See Section 5.

- Sample & Hold: Samples a signal at intervals and holds the value.

- Random / Noise modulator: Outputs random values at audio or sub-audio rate.

- Sequencer as modulator: Step sequencer outputs CV values to modulate parameters rhythmically.

- Macro controls: Single knob mapped to multiple parameters simultaneously with variable depth.

- Modulation matrix: Any source to any destination with depth control.

- Turing machine / generative: Algorithmic shift-register patterns driving modulation.

- Chaos / attractor modulator: Non-repeating mathematical attractor (Lorenz, logistic) drives modulation.

# 8. Types of Sequencers

Sequencers automatically play back a series of programmed events over time — notes, rhythms, and/or parameter changes — driving synthesizers and other instruments.

## 8.1 Step Sequencers

Divides a pattern into discrete steps, each containing note pitch, velocity, and duration data. Steps play back in order at a set tempo.

- Monophonic step sequencer: One note per step. Found in Roland TB-303, classic hardware.

- Polyphonic step sequencer: Multiple notes per step (chords). Found in Elektron Octatrack, Ableton Live.

- CV/Gate step sequencer: Outputs analog CV/Gate per step. Eurorack standard.

- Variable-length steps: Each step can have an independent gate length.

- Probability-based steps: Each step has a probability of firing. Elektron "p-locks".

- Parameter locks (p-locks): Per-step values for any synth parameter, not just pitch. Elektron innovation.

- Euclidean sequencer: Distributes a set number of hits evenly across a pattern of steps.

## 8.2 Pattern-Based Sequencers

- Pattern-based (DAW piano roll): Freely placed notes in time with variable lengths. Non-step-based.

- Drum machine sequencer: Dedicated to percussive patterns; one row per drum sound.

- Clip / scene launcher: Patterns assigned to clips and launched live (Ableton Live, Bitwig Grid).

- Song arranger: Chains patterns in sequence for full-song composition.

## 8.3 Melodic / Algorithmic Sequencers

- Random sequencer: Steps play in random order for generative music.

- Pendulum sequencer: Plays forward then backward (bouncing).

- Scale-quantized sequencer: Output notes quantized to a chosen musical scale.

- Generative sequencer: Uses algorithms (Markov chains, L-systems) to generate evolving patterns.

- Arpeggiator-as-sequencer: Notes derived from a held chord played in sequence (see Section 9).

- Pitch sequence vs. rhythm sequence: Independent sequences for pitch and gate/rhythm.

## 8.4 MIDI Sequencers

- Hardware MIDI sequencer: Standalone device (Roland MSQ-700, Korg SQ-10, Arturia BeatStep Pro).

- DAW MIDI sequencer: Piano roll and pattern editors in Ableton Live, Logic, FL Studio, Cubase.

- MIDI arpeggiator/sequencer plugins: Cthulhu, MIDI Polyphony, Unify.

## 8.5 CV/Gate Sequencers (Modular)

- Analogue Solutions Oberkorn, Make Noise René, Intellijel Metropolix

- Multi-channel CV sequencer: Outputs multiple simultaneous CV streams for polyphonic modular.

- Non-linear/matrix sequencer: 2D grid navigation (Make Noise René, Buchla 248).

# 9. Types of Arpeggiators

An arpeggiator automatically steps through the notes of a held chord in a specified pattern and tempo. Found in virtually every synthesizer and DAW.

## 9.1 Direction Modes

- Up: Notes play from lowest to highest pitch, repeating.

- Down: Notes play from highest to lowest pitch.

- Up/Down (bounce): Alternates direction at each end of the range.

- Down/Up: Starts high, bounces.

- Random: Notes selected randomly from the held notes.

- As played: Notes play in the order they were pressed on the keyboard.

- Chord: All notes play simultaneously (not true arpeggio; chord strumming).

- Alternating: Alternates two arpeggio patterns.

## 9.2 Octave Range

- 1 octave: Notes play within the span of the held chord.

- 2 octaves: Arpeggio spans two octaves up or down.

- 3–4 octaves: Wider sweeps for dramatic melodic lines.

- Octave up, then down: Rises through octaves then descends.

## 9.3 Rate / Division

- Fixed rate: Arpeggio steps at a fixed BPM-independent speed.

- Tempo-synced: Steps aligned to note divisions (1/4, 1/8, 1/16, 1/32, dotted, triplet).

- Gate length per step: Legato, staccato, or variable gate per arpeggio step.

## 9.4 Advanced Arpeggiator Features

- Note repeat: Repeats individual notes multiple times per step.

- Skip steps: Certain arpeggio positions skip, creating gaps.

- Velocity control: Accented steps at specific positions.

- Latch mode: Arpeggio continues after keys are released.

- MIDI capture: Records arpeggio output as MIDI notes to sequencer.

- Chord arpeggiator: Arpeggios constructed from chord inversions and voicings.

- Step sequencer hybrid: Arpeggiator combined with programmable step offsets.

## 9.5 Notable Arpeggiator Implementations

- Korg MS-10/20: Basic up/down arpeggiator.

- Roland Juno-106: Classic tempo-synced arpeggiator.

- Access Virus: Comprehensive arpeggiator with pattern sequencer mode.

- Novation Nova / SuperNova: Multi-octave, pattern-based arpeggiator.

- Native Instruments Massive: Step-arpeggiator with per-step pitch, rhythm, and velocity.

- Arturia Pigments: Pattern arpeggiator with polyphonic capabilities.

# 10. UI/UX Design Theory for Synthesizers

Synthesizer interface design balances learnability, expressiveness, and aesthetic appeal. Both hardware and software synths must communicate complex signal flows in an immediately usable way.

## 10.1 Core UI/UX Principles

- Affordance: Controls should suggest their function (knobs = rotate, sliders = slide, buttons = press).

- Feedback: Every interaction yields immediate visual and/or auditory response (knob moves, sound changes).

- Discoverability: Users should be able to find features without reading a manual.

- Consistency: Similar controls behave similarly throughout the interface.

- Mapping: Spatial arrangement of controls should reflect logical and signal-flow relationships.

- Mental model alignment: The interface should match how users conceptually understand synthesis.

- Learnability vs. efficiency: Beginners need guides; experts need fast access.

## 10.2 Signal Flow Visualization

- Left-to-right layout: Mirrors classic signal flow (OSC → FILTER → AMP → FX). Used in Minimoog, Moog One, Arturia designs.

- Top-to-bottom layout: Less common; used in some modular patching contexts.

- Block diagram overlay: Software synths sometimes show signal routing diagrams (u-he Hive, Surge XT).

- Patch cable visualization: Drag-and-drop virtual patch cables (VCV Rack, Softube Modular).

## 10.3 Hardware UI Conventions

- Knob-per-function: One physical knob per parameter for immediate tactile control. Minimoog, Roland SH-101.

- Dedicated panel sections: OSC, FILTER, ENV, LFO sections clearly labeled and grouped.

- LED indicators: Show LFO position, arpeggio step, MIDI activity.

- Color coding: Colored knobs or labels distinguish sections (e.g., Korg Minilogue xd).

- Silkscreened panel: Printed labels and graphics aid navigation without screen.

- Multi-function controls: Shared controls with mode buttons (reduces cost, increases complexity).

## 10.4 Software UI Conventions

- Skeuomorphic design: Mimics hardware appearance (wood panels, metal knobs). Arturia V Collection, u-he Diva.

- Flat/modern design: Clean, minimal, high-contrast UI without hardware metaphors. Vital, Serum.

- Hybrid design: Clean look with hardware-inspired interaction. Xfer Serum, Massive X.

- Resizable UI: Scales to different screen sizes and resolutions.

- Tooltips: Hover over controls for parameter names and current values.

- Contextual menus: Right-click menus for parameter options, MIDI learn, modulation assignment.

- Modulation visualization: On-screen display of modulation depth and routing (Serum mod arcs, Vital mod lines).

- Preset browser: Hierarchical preset categories, search, favoriting. Critical for workflow.

- Dark mode / light mode: Especially relevant for studio lighting conditions.

## 10.5 Accessibility and Ergonomics

- Knob size and spacing: Critical for hardware—knobs spaced for comfortable hand use without accidental touching.

- Screen readability: High contrast labels; readable at arm's length in stage conditions.

- Font size and clarity: Crucial in software for small plugin windows.

- Keyboard shortcuts: Power-user keyboard navigation in software synths.

- MIDI learn: Map any hardware controller to any parameter easily.

- Undo/redo: Essential for software instruments.

## 10.6 Performance-Oriented UI

- Dedicated performance controls: Pitch bend, mod wheel, ribbons, XY pads.

- Patch naming and organization: Clear preset management for live performance.

- Macro/quick controls: Assign complex parameter sets to single controls for live tweaking.

- Minimal-UI live mode: Simplified view hiding deep editing during performance.

- Touch screen optimization: Large targets, swipe gestures (software or touchscreen hardware).

# 11. Musical Genres Associated with Synthesizers

Synthesizers have shaped virtually every genre of modern music since the 1960s. The following covers the primary genres and their characteristic synthesizer usage.

## 11.1 Electronic Dance Music (EDM)

- Subgenres: House, Techno, Trance, Drum & Bass, Dubstep, Future Bass, Hardstyle

- Key synths: Roland TR-808/909 (drums), Roland TB-303 (acid bass), Xfer Serum (leads/supersaws), Native Instruments Massive (dubstep bass/wobbles)

- Example: Daft Punk – 'Get Lucky' (Minimoog bass); deadmau5 – 'Strobe' (Serum pads)

## 11.2 Ambient / New Age

- Minimalist, textural, slow-evolving pads and drones

- Key synths: Roland Juno-106, Yamaha DX7, Ensoniq ESQ-1, software granular synths

- Example: Brian Eno – 'Ambient 1: Music for Airports' (piano + tape delay); Aphex Twin – 'Selected Ambient Works Vol. II'

## 11.3 Synthwave / Retrowave

- 80s-inspired analog and digital textures, arpeggiated leads, gated reverb drums

- Key synths: Roland Juno-106, Jupiter-8, Oberheim OB-Xa, Arturia V Collection

- Example: Kavinsky – 'Nightcall'; Carpenter Brut – 'Turbo Killer'

## 11.4 Progressive Rock / Art Rock

- Complex arrangements using Mellotron, ARP, Moog for orchestral textures and solos

- Key synths: Minimoog, ARP 2600, Mellotron, Hammond organ with Leslie

- Example: Keith Emerson (ELP) – 'Karn Evil 9' (Moog solos); Rick Wakeman (Yes) – ARP and Minimoog

## 11.5 Krautrock / Experimental Electronic

- Motorik rhythms, sequenced synths, drones, unconventional structures

- Key synths: ARP 2600, Korg MS-20, early modular systems (Buchla, EMS VCS3)

- Example: Kraftwerk – 'Autobahn' (EMS Synthi); Tangerine Dream – 'Phaedra'

## 11.6 Hip-Hop / Trap / R&B

- Sampled and synthesized melodic elements, 808 sub-bass, vocal chops

- Key synths: Roland TR-808 (sub bass via pitch envelope), Yamaha DX7 (electric piano), Serum, LABS

- Example: Kanye West – 'Love Lockdown' (808 bass); Future – trap production with 808 subs

## 11.7 Industrial / EBM

- Harsh, distorted, rhythmically aggressive synthesizer sequences

- Key synths: Roland SH-101, Juno-106, sequential Prophet-5, Korg MS-20

- Example: Nine Inch Nails – 'The Becoming' (modular); Depeche Mode – 'Policy of Truth' (Oberheim)

## 11.8 Cinematic / Film Score

- Orchestral synthesis, hybrid scoring, textural underscore

- Key synths: Spectrasonics Omnisphere, Kontakt, Spitfire LABS, east west Hollywood Strings + synths

- Example: Hans Zimmer – 'Interstellar' score (custom synths, organs); Vangelis – 'Blade Runner' (Yamaha CS-80)

## 11.9 Acid / Electro

- Squelchy, resonant TB-303 bass sequences; drum machine patterns

- Key synths: Roland TB-303, TR-808, Cyclone Analogic TT-303 (clone)

- Example: Phuture – 'Acid Tracks' (the original acid house record, TB-303)

## 11.10 Pop

- Ubiquitous synth pads, arpeggios, leads across all eras

- Key synths: Roland Juno-106, DX7 (1980s), Serum/Sylenth1 (2010s), omnisphere

- Example: A-ha – 'Take On Me' (Juno arpeggio); Katy Perry – 'Teenage Dream' (contemporary pop synthesis)

# 12. Use Cases for Synthesizer Architectures

## 12.1 Subtractive Synthesis

- Classic bass lines (Minimoog-style, TB-303 acid)

- Lead synth melodies and solos

- Pad textures using PWM and chorus

- Brass and wind emulations via envelope shaping

- Percussive sounds (closed hi-hats from VCO noise + fast decay)

## 12.2 FM Synthesis

- Electric piano and Rhodes-like tones (DX7 patches)

- Bell and metallophone sounds

- Plucked string emulation

- Complex, inharmonic percussive sounds

- Shimmery digital pad textures

## 12.3 Wavetable Synthesis

- Evolving, morphing digital pads

- Modern EDM leads and supersaws

- Rhythmic, animated textures via wavetable scanning

- Hybrid textures combining wavetable with subtractive filtering

## 12.4 Sample-Based / ROMpler

- Realistic acoustic instrument emulation (pianos, strings, brass)

- Hybrid scoring (layered real instruments + synthesis)

- World instruments and ethnic sounds

- Sound effects and Foley

## 12.5 Physical Modeling

- Detailed acoustic instrument simulation (acoustic piano, violin, flute)

- Tonewheel organ simulation

- Non-existent "impossible" acoustic instruments

- Real-time performance expression (bowing pressure, embouchure)

## 12.6 Granular Synthesis

- Ambient drone and texture generation

- Time-stretching without pitch artifacts

- Glitchy, deconstructed sound design

- "Cloudscapes" and frozen audio textures

## 12.7 Modular Synthesis

- Generative and algorithmic music composition

- Experimental and academic sound research

- Non-standard tuning systems (just intonation, microtonal)

- Fully customized performance instruments

## 12.8 Virtual Analog

- All subtractive synthesis use cases with added digital stability

- Polyphonic analog-style pads and strings

- Live performance where tuning stability is essential

# 13. Virtual Analog (VA) Synthesizers

## 13.1 What Makes a Good Virtual Analog Synth

A high-quality VA synthesizer must accurately model the sound and behavior of analog circuitry while providing the stability and flexibility of digital instruments.

- Accurate oscillator models: Bandlimited waveforms (BLIT, BLEP, polyBLEP) to prevent aliasing. Slight oscillator drift and detuning for organic warmth.

- Filter modeling quality: True circuit-level modeling of classic filter topologies (Moog ladder, Sallen-Key, SEM). Nonlinear saturation and self-oscillation behavior.

- Zero Delay Feedback (ZDF) filters: Eliminates the one-sample delay in naive digital filter implementations for more accurate resonance behavior.

- Oversampling: Internal processing at 2x–8x sample rate to reduce aliasing artifacts, especially at high resonance and distortion.

- Unison / voice stacking: Multiple oscillators per voice with individual detuning and spread.

- Analog-style modulation: Smooth, musical modulation with no stepping or zipper noise.

- Thermal drift modeling: Subtle pitch drift per voice for analog warmth. Applied per oscillator independently.

- Component-level simulation: Models resistor tolerances, capacitor values, and transistor behavior.

- High-quality voice architecture: Per-voice effects, envelope, and LFO for true polyphonic expression.

- Low latency: Efficient DSP for real-time performance without noticeable latency.

## 13.2 Top Virtual Analog Synthesizer Brands

- u-he (Germany): Diva (industry-leading analog modeling), Repro-1, Repro-5, Hive. Known for extreme accuracy and warmth.

- Arturia (France): V Collection — Minimoog V, Jupiter-8 V, OB-Xa V, SEM V, CS-80 V. Best-in-class vintage hardware emulations.

- Native Instruments (Germany): Massive (classic EDM VA), Monark (Minimoog model), Pro-53 (Prophet-5 emulation).

- Clavia / Nord (Sweden): Nord Lead series — hardware VA pioneering since 1994. Known for performance-focused design.

- Access (Germany): Virus TI series — hardware VA with extensive modulation and effects.

- Roland (Japan): JP-8000, JP-8080 (hardware VA with supersaw). Roland Cloud software emulations of SH-101, Juno-106, Jupiter-8.

- Waldorf (Germany): Blofeld (hardware VA/wavetable hybrid), Quantum, M.

- TAL Software (Switzerland): TAL-U-No-LX (Juno-60 emulation), TAL-Bassline (TB-303). Free and premium options.

- Softube (Sweden): Model 82 (TB-303 emulation), various analog circuit models.

- Surge XT (open source): Comprehensive VA with multiple filter types, free and open source.

# 14. How DSP Works in Synthesizers

Digital Signal Processing (DSP) is the mathematical manipulation of audio signals represented as sequences of numbers. In synthesizers, DSP generates, shapes, and transforms audio in real time.

## 14.1 Core DSP Concepts

- Sample rate: Number of audio samples per second (e.g., 44,100 Hz = 44.1 kHz). Defines the maximum representable frequency (Nyquist = sample rate / 2).

- Bit depth: Number of bits per sample (16-bit = 96 dB dynamic range; 32-bit float = ~1,528 dB theoretical). Internal processing typically uses 32-bit or 64-bit floating point.

- Nyquist theorem: A signal must be sampled at more than twice its highest frequency to be accurately represented. Aliasing occurs when this rule is violated.

- Aliasing: Spurious frequencies that appear when a signal contains content above Nyquist. Must be prevented in oscillator generation via bandlimiting.

- Oversampling: Processing at a multiple of the sample rate (2x, 4x, 8x) to push aliasing products above Nyquist, then downsampling with anti-aliasing filters.

- Fixed-point vs. floating-point: Fixed-point arithmetic (integers) is used on dedicated DSP chips for efficiency; floating-point is standard in modern CPU-based synthesis.

## 14.2 Oscillator DSP

Example: Generating a 440 Hz sawtooth wave at 44,100 Hz sample rate.

- Phase accumulation: Each sample, add a phase increment (440 / 44100 = 0.00997... radians/sample × 2π). When phase exceeds 1.0, wrap around.

- Naive sawtooth: Output = 2 × phase - 1. Simple but produces aliasing due to discontinuity at phase reset.

- PolyBLEP correction: At the phase reset discontinuity, apply a polynomial correction to smooth the edge. Eliminates aliasing without oversampling. Most efficient VA method.

- BLIT (Bandlimited Impulse Train): Generate a train of sinc pulses bandlimited to Nyquist, then integrate to produce the desired waveform.

- Wavetable: Look up sample value in a stored table using the phase value as an index. Cubic or linear interpolation between table entries.

## 14.3 Filter DSP

Example: Implementing a resonant low-pass filter.

- Biquad IIR filter: Second-order recursive filter. Defined by five coefficients (b0, b1, b2, a1, a2). Calculated from cutoff frequency and Q (resonance) using bilinear transform.

- Bilinear transform: Maps an analog prototype filter (s-domain) to a digital filter (z-domain) by substituting s = 2/T × (z-1)/(z+1). Preserves frequency response up to Nyquist.

- Resonance / Q: High Q makes the filter peak sharply at the cutoff. Implemented by feeding back the output with a gain factor.

- Self-oscillation: When resonance is high enough, the filter becomes an oscillator at its cutoff frequency. Modeled by allowing the feedback loop to sustain.

- Nonlinearity (saturation): Real analog filters saturate, adding harmonics. Modeled with tanh() or soft-clipping functions applied within the filter loop.

- Zero Delay Feedback (ZDF): Uses trapezoidal integration to eliminate the one-sample delay in standard IIR structures, producing more accurate resonance and stability at extreme settings.

## 14.4 Envelope DSP

- State machine: Envelope generator implemented as a state machine cycling through ATTACK → DECAY → SUSTAIN → RELEASE states.

- Exponential curves: Attack and decay curves are often exponential (matching human perception of loudness) computed as: output = target + (output - target) × coefficient.

- Linear vs. exponential: Linear envelopes change at constant rate; exponential approach a target asymptotically.

- Sample-accurate triggering: High-quality synths implement per-sample envelope updates for precise timing at audio rate.

## 14.5 Reverb DSP

- FDN (Feedback Delay Network): Multiple delay lines with feedback matrix. Produces diffuse reverb tails. Schroeder/Moorer designs.

- Convolution reverb: FFT-based convolution of dry signal with recorded impulse response. Produces photorealistic room simulation but is computationally intensive.

- Algorithmic reverb: Combines delay lines, all-pass filters, and comb filters for synthetic reverb. More efficient than convolution.

# 15. DSP of Individual Building Blocks

## 15.1 VCO / Oscillator DSP

Phase accumulator: phase += (frequency / sampleRate). Output waveform computed from phase. Sawtooth = 2*phase - 1. Square = phase < duty ? 1.0 : -1.0. Triangle = 1 - 4*abs(phase - 0.5). PolyBLEP correction applied at discontinuities to eliminate aliasing.

## 15.2 VCF / Filter DSP

State Variable Filter (SVF) in digital form — Chamberlin topology: low = low + f * band; high = input - low - q*band; band = f * high + band; notch = high + low. Where f = 2*sin(π*cutoff/sampleRate) and q = 1/resonance.

## 15.3 VCA / Amplitude DSP

Multiply signal by envelope output: output = signal * envelopeLevel. VCA gain is linear in voltage but logarithmic in dB — good implementations use dB-linear gain curves for musical response.

## 15.4 Envelope Generator DSP

State machine with per-sample updates. Attack: level += (1.0 - level) * attackCoefficient. Decay: level += (sustainLevel - level) * decayCoefficient. Release: level += (0.0 - level) * releaseCoefficient.

## 15.5 LFO DSP

Same as oscillator but at sub-audio rates. Phase accumulation: phase += (rate / sampleRate). For tempo sync: rate = bpm/60 × divisor. Sine output = sin(2π × phase). Often computed at a reduced rate (control rate) for efficiency.

## 15.6 Ring Modulator DSP

Multiplication of two signals: output = signal_a * signal_b. Produces sum and difference sidebands (output frequencies = |f1 ± f2|). If the carrier is a sine wave at frequency fc, a pure tone at f1 produces tones at f1+fc and f1-fc.

## 15.7 Chorus DSP

Short delay line (5–30 ms) with LFO-modulated delay time, mixed back with dry signal. output = dry + wet * delayBuffer[readPos + LFO * depthSamples]. Multiple delay lines at different phases create richer chorus.

## 15.8 Delay / Echo DSP

Circular buffer of audio samples. Write current sample to buffer; read from buffer at [writePos - delayTimeSamples]. Mix feedback: buffer[writePos] = input + buffer[readPos] * feedbackGain. Modulated delay time for flanging/chorus.

## 15.9 Waveshaper DSP

Applies a nonlinear transfer function to each sample. Soft clipping: output = (3/2) * input * (1 - input² / 3) for |input| < 1. Hard clipping: output = clamp(input, -1, 1). Tanh saturation: output = tanh(gain * input).

## 15.10 Granular Synthesis DSP

Maintain a buffer of audio samples. Spawn 'grains' — small windows of samples — with parameters: start position, duration, pitch (playback speed), amplitude envelope (e.g., Hann window), pan position. Overlap and add grains together. Control density (grains/second), position scatter, pitch scatter.

# 16. Software Architecture of Synthesizers

A software synthesizer must efficiently generate audio in real time within strict timing constraints imposed by the audio driver. The architecture determines performance, flexibility, and sound quality.

## 16.1 High-Level Architecture

- Audio callback (DSP thread): The host (DAW or OS audio driver) calls a render function at regular intervals (buffer size, typically 64–512 samples). This thread must complete in less than the buffer duration or audio will glitch (xrun).

- MIDI/control thread: Receives incoming MIDI messages, user input, and automation data. Passes to DSP thread via lock-free queues.

- UI thread: Renders the graphical interface. Completely separate from DSP to avoid blocking audio.

- Lock-free inter-thread communication: FIFO queues (ring buffers) pass messages between threads without mutexes to avoid priority inversion.

## 16.2 Voice Architecture

- Monophonic: Single voice. Simple but expressive (classic Minimoog design).

- Polyphonic: Multiple simultaneous voices, each with its own oscillators, filters, and envelopes.

- Voice stealing: When all voices are in use and a new note arrives, an active voice is "stolen" (oldest, quietest, or lowest-priority voice).

- Unison: Multiple voices per note, detuned and spread stereo.

- Per-voice effects: Each voice processes through its own FX chain before mixing.

## 16.3 Signal Graph / Module Graph

- Fixed pipeline: Preset order of processing (OSC → FILTER → AMP → FX). Efficient, less flexible.

- Flexible signal graph: Modules connected in any order (modular synths, Bitwig Grid, Max/MSP). More overhead but fully reconfigurable.

- Block processing: Audio processed in blocks (e.g., 64 samples at a time) rather than sample-by-sample for CPU efficiency.

## 16.4 DSP Engine Implementation

- Sample rate processing: Every module computes its output for each audio sample.

- Control rate processing: LFOs and envelopes computed at a fraction of audio rate (e.g., every 32 samples) for efficiency.

- SIMD vectorization: Modern CPU SIMD instructions (SSE, AVX, NEON) process multiple samples in parallel.

- Multi-threading: Multiple voices processed on separate CPU cores.

- Fixed vs. floating point: Software uses 32-bit float; hardware DSP chips may use fixed-point.

## 16.5 Plugin Formats

- VST2 / VST3 (Steinberg): Most widely supported. VST3 adds improved parameter automation, bus management, and MIDI-CI support.

- AU (Audio Units, Apple): macOS/iOS native format. Required for Logic Pro and GarageBand compatibility.

- AAX (Avid): Required for Pro Tools.

- CLAP: Modern open-source plugin format with improved threading and parameter handling.

- Standalone: Runs without a DAW host, directly on the OS audio driver (ASIO on Windows, CoreAudio on Mac).

## 16.6 Real-Time Constraints

- Buffer size: Smaller buffer = lower latency but higher CPU load. Typical performance: 256 samples at 44.1 kHz ≈ 5.8 ms.

- Priority: DSP thread runs at real-time priority to prevent preemption by the OS.

- Memory allocation: No malloc/new in the DSP thread. All memory allocated at initialization.

- Denormals: Sub-normal floating-point numbers cause severe performance degradation. Mitigated with FTZ (Flush-to-Zero) flags.

- Parameter smoothing: Raw parameter changes cause clicks. Interpolate over 1–10 ms.

# 17. Emulating Hardware Synths with DSP

Creating convincing hardware emulations requires deep analysis of the original circuit's behavior and implementation of corresponding mathematical models in DSP.

## 17.1 Analysis and Measurement

- Circuit analysis: Study original schematics to understand signal path, component values, and nonlinearities.

- SPICE simulation: Simulate the analog circuit with SPICE (LTspice, ngspice) to characterize component-level behavior.

- Audio measurement: Measure the actual hardware using test tones, swept sine, noise analysis. Record frequency response, harmonic distortion, noise floor, and filter behavior.

- Impulse response capture: Record how the hardware responds to a brief impulse for convolution-based effects.

- Component tolerance analysis: Real analog hardware has component variations (1–5% resistor tolerances). These contribute to voice-to-voice variation and "analog warmth."

## 17.2 Oscillator Emulation

- Waveform shape matching: Analog sawtooth waves are not perfect; they have slew rate at the discontinuity. Model this with exponential or linear rise before the reset.

- Oscillator drift: Analog VCOs drift with temperature. Model with slow random modulation (pink noise) applied to pitch — ±1–5 cents over seconds.

- Slew / waveform asymmetry: Some analog oscillators produce asymmetric triangles or slightly rounded sawtooth edges. Capture via measurement and model accordingly.

- Cross-modulation behavior: Analog hard sync produces ringing at the sync point. Model this transient realistically.

## 17.3 Filter Emulation

- Moog ladder filter model: Transistor ladder with four cascaded one-pole stages. Each stage: output = output + f*(tanh(input/2) - tanh(output/2)). Feedback: input to first stage = input - resonance * tanh(output_stage4). Key: nonlinear tanh in each stage for saturation.

- Component-level modeling: Model individual transistors, resistors, capacitors with their exact values from the schematic.

- Resonance limiting: In real hardware, high resonance reduces output level. Implement resonance compensation to match.

- Cutoff frequency tracking: Real analog filters have slight pitch tracking errors. Replicate this characteristic.

- ZDF (Zero Delay Feedback): Eliminate one-sample delay artifacts using implicit Euler or trapezoidal integration.

## 17.4 Amplifier and Saturation Emulation

- VCA nonlinearity: Real VCAs (CA3080 OTA, etc.) have characteristic soft clipping curves. Model with measured transfer functions.

- Output stage saturation: Analog output stages add subtle harmonic distortion. Apply tanh() or polynomial waveshaping to final output.

- Noise floor: Real analog has thermal noise. Add bandlimited noise at appropriate levels (typically -90 to -100 dBFS).

## 17.5 Voice and Polyphony Emulation

- Voice allocation: Original polyphonic synths (Prophet-5, Jupiter-8) use specific voice allocation algorithms. Replicate oldest-note stealing, note priority, etc.

- Voice-to-voice variation: Each voice in a real polyphonic synth has slightly different tuning and filter characteristics. Apply random per-voice offsets.

- Portamento behavior: Match the analog glide rate curve (linear in Hz vs. exponential in pitch).

## 17.6 Notable DSP Emulation Techniques

- u-he Diva approach: "Analog modeled" — proprietary circuit simulation running in real time with oversampling. Multiple filter models selectable.

- Arturia TAE® (True Analog Emulation): Proprietary analog modeling technology used across the V Collection. Incorporates oscillator and filter circuit models.

- Direct Form vs. Transposed Form: Different IIR filter structures have different numerical stability properties. Choose based on the hardware being modeled.

- Anti-aliasing for distortion: Any nonlinear process (waveshaping, filter saturation) creates aliasing. Oversample before nonlinearity, then downsample.

## 17.7 Summary Workflow for Hardware Emulation

- 1. Obtain and study original schematics

- 2. Build SPICE simulation; verify against measurements

- 3. Identify key nonlinearities (filter saturation, VCA clipping)

- 4. Implement DSP building blocks with ZDF and oversampling

- 5. Tune free parameters (component values, tolerance spreads) against hardware measurements

- 6. A/B test against original hardware, iterate

- 7. Optimize for CPU efficiency (SIMD, control-rate processing, profile-guided optimization)

— End of Document —